| BOARD | 6602 | 6601 | M-Series | E-Series | PC-TIO-10 |
| --- | --- | --- | --- | --- | --- |
| Counter ASIC | NI-TIO | NI-TIO | DAQSTC2 | DAQSTC | AMD9513 |
| Channels | 8 | 4 | 2 | 2 | 10 |
| Resolution | 32-bits | 32-bits | 32-bits | 24-bits | 16-bits |
| Timebases | 80 MHz, 20 MHz, 100 kHz | 20 MHz, 100 kHz | 80 MHz, 20 MHz, 100 kHz | 20 MHz, 100 kHz | 5 MHz, 1 MHz, 100 kHz, 10 kHz, 1 kHz, 100 Hz |
| prescalers | X8, X2, none | X8, X2, none | X8, X2, none | none | none |
| Max Freq w/ prescalers | 80 MHz 125 MHz | 20 MHz 60 MHz | 80 MHz N/A | 20 MHz N/A | 7 MHz N/A |
| DIO | up to 32 bits | up to 32 bits | up to 48 bits | up to 32 bits | 16 bits |
| DMA | 3 channels | 1 channel | 6 channels | 3 channels | None |
| RTSI | Yes | Yes | Yes | Yes | No |
| Updown counter | Yes | Yes | Yes | Yes | No |
| Encoders support | Yes | Yes | Yes | with external IC | No |
| Debounce Filters | Yes | Yes | Yes | No | No |
| Two Signal Edge Separation | Yes | Yes | Yes | No | No |
| Start Trigger (hardware arm) | Yes | Yes | Yes | No | No |
| Simulataneous Counter Arm | Yes | Yes | Yes | No | Yes (upto 5) |
| Continuous Buffered Input | Yes | Yes | Yes | Yes | No Buffered Input |
| Dynamic Pulse Train Freq Change | Yes | Yes | Yes | Yes | No |
| New LV layer | Yes | Yes | Yes | Yes | No |
| Old LV layer | Not recommended | Not recommended | No | Yes | Yes |